ComView SCTU

ComView SCTU (Serial Communications Test Utility) provides many useful

features for engineers, technicians, programmers, or anyone who works with

serial communications devices.

FEATURES:

Displays up to four simultaneous interrupt-driven terminal windows.

Received control characters are displayed, for example,

carriage return =  [CR]

All port addresses and interrupts are configurable.  (Works with

non-standard I/O cards)

Status of handshake lines are shown in real-time on all ports.

Programmable function keys allow easy interaction with various devices:

bar-code readers, ASCII terminals, etc.

Intercept cable can be built to allow monitoring both receive and

transmit channels of the communications flow between two devices

simultaneously.

Port merge window displays data received from all ports in sequence.

Data received from any port or the merge window may be captured to a file.

File viewer allows captured files to be viewed with control characters

displayed.  Captured files from merge port show text in different colors

for each port.

SYSTEM REQUIREMENTS:

IBM PC or compatible with 512K RAM

One or more serial ports using 8250, 16450, or 16550 or compatible UARTs

DOS 3.1 or greater

INSTALLATION:

All program functions are integrated into a single executable file,

"COMVIEW.EXE"  You may copy this file to a utilities directory on your

hard disk.  By having the directory in the PATH statement in your

AUTOEXEC.BAT file you will have access to Comview SCTU from any

directory.

There is no setup required before using Comview SCTU.  Simply type

"COMVIEW" from the command prompt.  The program will start with default

configuration settings.  When the program is exited an additional

file "COMVIEW.CFG" will be creating in the same directory that contains

"COMVIEW.EXE"  The "COMVIEW.CFG" file is written each time the program

Page 1

is exited with the current configuration settings.  When the program is

started again the "COMVIEW.CFG" file is read to recall the previous

settings.

OPERATION:

To start the program enter "COMVIEW" at the command prompt.

*** SHAREWARE ***                                     *** SHAREWARE ***

This is the shareware version.  At this point during program operation

a notice screen provides information about registering the software.

To continue on with program operation press a key or otherwise follow

the instructions provided on the screen.

*** SHAREWARE ***                                     *** SHAREWARE ***

The terminal screen appears with a title bar at the top of the screen.

This title bar shows the program name, "ComView SCTU", the version

number and the date the program was compiled.

One, two, three, or four terminal windows are shown below the title bar.

Each terminal window contains a status bar at its bottom line.

STATUS BAR

Capture status - An asterisk is shown at the far left of the status

bar while received data is being captured to a file.

Handshake lines

CD is shown when the Carrier Detect line is active

RI is shown when the Ring Indicator line is active

DSR is shown when the Data Set Ready line is active

CTS is shown when the Clear To Send line is active

Received character count (This reading is replaced by the error

message "Send Err" if an error occurs while data is sent, "Rcv Err"

appears if an error occurs while receiving data)

Port or device name

Hold status "<H>" appears on the status bar when the port is in the

display hold mode.

Handshake mode

H=N is shown when no handshaking is enabled

H=X is shown when XON/XOFF handshaking is enabled

Page 2

Interrupt (I=XX where XX is the IRQ number)

Port settings

(If the port failed initialization, the message "Can not set UART

at XXXX" will appear if a serial port is not found at the specified

address.  The message "Invalid port cfg" will appear if the baud,

word size, or stop bit values are unacceptable.  The message

"Invalid IRQ" will appear if the IRQ value is unacceptable.  The

message "IRQ conflict" will appear if an attempt is made to set

a window to an IRQ in use by another window.  The message

"Port In Use/No UART at XXXX" will appear if the DTR line of the UART

is high.  This may indicate that another application is using the

selected port, or there may be no serial port available at the

selected address.)

Address (the address is shown in hexadecimal)

Baud rate

Parity ("N" for None, "E", for Even, or "O" for Odd)

Data bits ("7" or "8")

Stop bits ("1" or "2")

MERGE WINDOW

The display lines between the bottom terminal status bar and the

function key labels is used to show the received data from all of

the individual terminal windows.  This is useful to observe data

sequences from multiple input ports.

FUNCTION KEY LABELS

The second lowest line of the display is used to show labels for the

function key assignments.

PROGRAM COMMANDS

The bottom line of the display shows program commands available.

All of the commands are available at any time, however not all of

the commands are shown at once.  Press the right cursor movement

key to see the second set of commands.  The left cursor movement

key will return the display of the first set of commands.

Up/Down  The Up and Down arrows on the keyboard allow selection of

the current terminal window.  The port or device name on the

current terminal is highlighted.  Any ASCII keys typed on

the keyboard are transmitted out the port of the current

window.

Page 3

Alt-P    Pressing Alt-P replaces the lower two lines of the display

with prompts for IRQ: XX Addr: XXXX Baud: XXXXXX Parity: X

Word: X Stop: X Handshake: X Color1: XX Color2: XX

Label: XXXXXXXXXXXXXXX  The values may be changed to alter

the port settings for the current window.

Alt-F?   Pressing any key Alt-F1 through Alt-F10 allows changing the

function key assignment for that function key.  The lower two

lines of the display are replaced with prompts, Enter FX Key

Label:  XXXXX Command: XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX.  A label

can be entered to display on the screen the purpose of the

function key.  A command can then be defined for the key.

Control characters can be entered as "^M" for control-M.

Special characters can be entered in hexadecimal format by

preceding the hexadecimal digits with the percent sign.  For

example, "%7F" would specify the character 7F in hexadecimal.

Alt-N    Pressing Alt-N allows changes to the general configuration.

The lower two lines of the display are replaced with the

input prompts, Number of Ports (1 - 4)? X  Border Color 1: XX

Border Color 2: XX  The number of terminal windows can be

selected and the display colors for the border lines on the

display can be changed.

Alt-X    Pressing Alt-X exits ComView SCTU.

*** SHAREWARE ***                                     *** SHAREWARE ***

This is the shareware version.  At this point during program operation

a notice screen provides information about registering the software.

To continue on with program operation press a key or otherwise follow

the instructions provided on the screen.

*** SHAREWARE ***                                     *** SHAREWARE ***

The DOS command prompt then returns.

Alt-C    Pressing Alt-C clears all data from current terminal window.

The cursor location is moved to the upper left corner for

the window.

Alt-S    Pressing Alt-S shuts down communications on all terminal

windows to allow repositioning the status bar of the current

window.  The Up and Down cursor movement keys are then used to

move the status bar to the desired position.  Pressing ENTER

completes the operation and resumes the communications.  By

selecting each window and resizing each window the entire

screen may be rearranged.

Alt-H    Pressing Alt-H turns on or off the display hold mode.  When

the display hold mode is on the text display for the window

is frozen.  Received characters are not shown.  Note however

if the port is being captured to a file received data is

still captured.

Page 4

PgDn     Pressing PgDn starts or stops the capture mode for the

selected port.  When starting the capture the filename will

be prompted for at the bottom of the display.

Alt-M    Pressing Alt-M starts or stops the capture mode for the

merge window.  When starting the capture the filename will

be prompted for at the bottom of the display.  An additional

file will also be created with the extension ".MRG"  As the

merge capture file is written, to codes are written to the

".MRG" file to identify which port the characters in the

capture file were received from.  This allows the file

viewer to show the merge file with text in different colors

based on the port that the text was received from.

Alt-I    Pressing Alt-I will cause a hard initialization of a port.

If a window is selected showing "Port In Use/No UART at XXXX"

this will force an attempt at initializing the port.  Note if

another application has control of the port this may cause the

computer to lock up or behave abnormally.

FILE VIEWER:

An additional program file "CVFILE.EXE" is provided to allow easy

viewing of captured files.  The viewer may be started by typing

"CVFILE filename" at the DOS prompt.  If a filename is specified at

the command prompt it will be displayed automatically, otherwise the

file selection display will appear to allow selection of the file from

the current directory.

While viewing a file Alt-F may be pressed to select a different file.

Typical editing keys Up, Down, Home, and End are used to navigate the

file.  If the file has an accompanying "*.MRG" file text colors identify

the port specific text was received from.  Alt-X may be pressed to exit

the viewer.

Page 5

SERIAL PORT TESTING

+-------------------------------+

| ********** NOTE: ************ |

|                               |

| Incorrect serial connections  |

| can damage computer equip-    |

| ment and peripherals. Only    |

| qualified electronics tech-   |

| nicians should perform com-   |

| munications testing.          |

| Wilson Computer Services will |

| assume no responsibility for  |

| damages due to use or misuse  |

| of the ComView SCTU program   |

| and documentation either      |

| directly or indirectly.       |

|                               |

| ***************************** |

+-------------------------------+

A useful test device can be built for identifying and testing unknown

serial ports for IBM compatible computers.  Most operating systems software

refer to these serial ports as COM1, COM2, COM3, or COM4.  These connections

are usually located on the back of the computer, mounted directly on the

chassis, on expansion cards open to the back of the chassis, or a combination

of both. These ports are usually either 9-pin male or 25-pin male connectors.

It is common for a computer to have a mixture of these; often an expansion

card will have a 25-pin connector and a 9-pin connector. Adapters are readily

available so that any serial device can be plugged into either type of port.

It is common for a computer to have three or four of these connections with

no identifying markings. Most software can be selected to operate any

particular serial device on various COM ports. However it can be a difficult

task to attempt a trial-and-error solution to connecting serial devices with

specialized software.

The test device this discussion is leading up to is called a loopback

tester. Simply put, it sends any output from the serial port it is plugged

into directly back into that same serial port. Diagrams for constructing

this device for the two common serial ports follow:

Page 6

25-Pin                              9-Pin

-----------------                   -----------------

+--  2  <-- TXD                     +--  3  <-- TXD

|                                   |

+--  3  --> RXD                     +--  2  --> RXD

+--  4  <-- RTS                     +--  7  <-- RTS

|                                   |

+--  5  --> CTS                     +--  8  --> CTS

+--  6  --> DSR                     +--  6  --> DSR

|                                   |

+-- 20  <-- DTR                     +--  4  <-- DTR

Parts Required: A 9-pin or 25-pin female connector. Small wires to

solder to it. A connector shell is also helpful to make the assembly

easier to handle.

Note: Rather than building both connectors it is time saving to just build

the 25-pin one and use a 9/25-pin adapter with it when testing 9-pin ports.

The actual data loop-back is accomplished with only one wire, the wire

between pins 2 and 3. This ties together the transmit data and the receive

data lines. The other jumpers are for handshake checking. Some serial

ports have these handshake lines jumpered internally so these connections

are not necessary. It is generally best to provide the handshake lines so

the loopback will work on any serial port.

If the serial ports on a particular computer do require handshake lines

it makes it easy to identify an unknown port with the loopback test.

Just exit any programs to get to a DOS prompt, such as "C:\>", then type

the line below followed by pressing the "ENTER" key.

ECHO HELLO > COM1

This attempts to send the word "hello" out the COM1 port. If nothing is

attached to this port and the port requires handshake lines the error

message which follows will appear on the screen.

Write fault error writing to device COM1.

Abort, Retry, Ignore, Fail?

Press "A" to abort, and the DOS prompt will appear. Attach the

loopback tester to the port and repeat the test. This time if the

correct port was selected the error message will not appear and instead

the DOS prompt will return immediately. To identify COM1 simply try

plugging the loopback tester into each available serial port and

repeat this test until the error does not occur. The port the tester

is plugged into can then be labeled COM1. Repeat the above procedures

for COM2, simply substitute the word COM2 for COM1. For DOS versions above

version 3.3 this can also be used to identify COM3 and COM4.

Page 7

Further testing of computer serial ports with a loopback tester can be

accomplished using the ComView SCTU program.

For loopback testing any baud rate, parity, data bit, and stop bit settings

will do fine.  For testing serial ports for use with particular devices

it is desirable to set the parameters as they will normally be used.

Simply plug the loopback tester into the COM port in question.  The CTS

and DSR handshake lines will show as active for the appropriate terminal

window of the ComView SCTU program.  Make sure the correct window is

selected for keyboard output using the up or down cursor movement keys.

Type on the keyboard.  Any characters typed should show up in the terminal

window because they are being sent out of the serial port and immediately

returned to the computer through the loopback tester.  Remove the loopback

tester and again type on the keyboard.  The characters typed will not show

in the terminal window because they are not returning to the serial port.

If a serial port being tested does not work the screen may show different

characters from the ones being pressed on the keyboard or not show any

characters with the loopback attached.

PERIPHERAL TESTING AND OPERATION

The ComView SCTU program can be used to test and operate various serial

devices.  For example, a computer with a Hayes compatible modem installed

internally or attached to a serial port can be controlled using ComView

SCTU.  Simply select the appropriate port address and interrupt for one

of the terminal windows.  Type "AT" and ENTER.  The modem should respond

with "OK[CR]" or something similar.

Peripheral devices should always be connected to the computer using the

appropriate cables provided by or recommended by the peripheral

manufacturer.

Bar code readers and other serial input devices may be attached to the

computer.  The appropriate terminal window when properly configured will

show data received from the device.  Commands may be issued to the device

simply by typing at the terminal window.  The programmable function keys

are useful to maintain complex commands which can then be sent to the

device with a single keystroke.

ASCII terminal units can be tested easily.  Any character keys typed at

the ComView SCTU terminal window should appear on the ASCII terminal unit.

Any character keys typed at the ASCII terminal unit should appear in the

ComView SCTU terminal window.

Serial printers can also be tested.  Serial printers normally buffer data

received so the printer may not print each character as it is typed in the

ComView SCTU terminal window.  Pressing carriage return or linefeed, Ctrl-J,

will normally cause a serial printer to printout everything in its buffer.

Serial printers also normally manipulate the handshake lines to provide

indications for various conditions such as off-line and paper out.

The handshake indicators on the status bar of the terminal window can be

Page 8

observed to verify that the printer is providing these signal changes when

the paper is removed and the printer is placed off-line.

Other devices such as weight indicators or temperature monitors may provide

serial outputs that are either continuous or provided when some condition

initiates it.  If a the device provides a continuous output the output can

monitored in the ComView SCTU terminal window.  As the window fills up with

data the display will scroll.  Some devices may have an output port, such

as a printer port, which is normally idle but outputs data when a button

is pressed on the device.  This can be observed by watching the ComView SCTU

window as the print button is pressed.  The output data will show up on the

terminal window.  Some devices may operate in on-demand mode.  A code such

as Ctrl-E can be pressed at the terminal window to initiate the output.

The data will then be sent to the computer.

INTERCEPT TESTING

It is often very useful to analyze the communications between two serial

devices as they are operating together.  This can be accomplished with an

intercept cable.  A separate computer, not involved in the communications

in question, must be available to run ComView SCTU.  The block diagram

below illustrates this operation.

+------+                    +------------+

|      |                    |            |

|  PC  +------------+-------+ Peripheral |

|      |            |       |            |

+------+            |       +------------+

|

+---------+  |

|   PC    |  |

| ComView +--+

|  SCTU   |

+---------+

The PC running ComView SCTU can be used to observe the data flow between

the two devices.  A 25-pin or 9-pin ribbon cable with several connectors

(male and female) is needed.   One end of the existing cable can be

unplugged.  The ribbon cable is inserted to provide the connection between

the two devices while also providing additional connectors to allow

connection of a cable to the PC running ComView SCTU.  There are two

types of intercept cables which can be constructed.

PC Running ComView SCTU                             Test End

-----------------------                       -------------------

9-Pin   25-Pin                               25-Pin   9-Pin

-----   ------                   switch      ------   -----

O------  2       3

2       3  --------------------O

O------  3       2

5       7  ---------------------------------  7       5

Page 9

This cable allows using only one of the terminal windows to monitor both

data received by and data transmitted from the peripheral.  However, only

one side of the conversation may be viewed at a time.  The switch on the

cable allows the selection of which side of the conversation is to be

viewed.  ComView SCTU allows a more versatile monitoring approach.

PC Running ComView SCTU                             Test End

-----------------------                       -------------------

Port A

9-Pin   25-Pin                               25-Pin   9-Pin

-----   ------                               ------   -----

2       3  ---------------------------------  2       3

+------------------------  3       2

|

5       7  ---------)--+--------------------  7       5

|   |

Port B            |   |

9-Pin   25-Pin       |   |

-----   ------       |   |

2       3  --------+   |

|

5       7  ------------+

This approach requires two COM ports on the PC running ComView SCTU.  It

allows both sides of the conversation with the Peripheral to be viewed

simultaneously in separate terminal windows.  The port merge window shows

both sides of the conversation in one window in the proper sequence.

Page 10